|  |  |  | TERMO DE ACEITE TÉCNICO |  |  |
| --- | --- | --- | --- | --- | --- |
| Solicitante: ATI - Agência de Tecnologia da Informação do Estado do Piauí |  |  |  |  |  |
| NOC/Operadora: Leonardo Martins |  |  |  | Técnico Instalador: Jean Polini Contato: 11 97701-2709 Email: jeanpolinitelecom@gmail.com |  |
| ATI: Weslley Souza |  |  |  | Responsável/cliente: Maria Semeão de Jesus |  |
| Orgão: SEDUC | Unidade: Unidade Escolar José Ricardo de Sousa |  |  | Data Instalação 1/09/2018 | Data Ativação 1/09/2018 |
| Operadora Global Eagle |  | VSAT-ID ATI-ES-0053-IND |  | Porta kbps 1024/256 | Tipo Indoor |
| Piauí, 1 de Setembro de 2018 1 – Local da Instalação. 2 – Configurações do Modem # device show System Information: Name : ATI-ES-0053-IND Location : PI Contact : System Up time : 0 days, 01:13:57 CPU Load : 12% System time(UTC) : 01 September 2018 19:42:56 Broadcast Message : not set HW: Model : SatLink 2000 HW ID : 120033 Main board ID : 120026 R6.2 MAC addresses: Ethernet (LAN) : 00:20:0e:10:85:b4 Satellite (DVB) : 00:20:0e:10:85:b4 # dvb tx show Satellite (DVB) TX Configuration -------------------------------- Auto start : Enabled IDU Output Power : -20 dBm IDU Max Output Power: 0.0 dBm Default CW Frequency: 0.000000 GHz ATM mode : VC-Mux Header Compression : None Satellite (DVB) Transmitter Status -------------------------------- State : On (DVB-RCS2) IDU Output Power : -19.9 dBm Es/No : 14.5 dB Header Compression : Disabled Timing correction : -132 us (249431 us) Frequency correction: 420 Hz # dvb rx show Satellite (DVB) RX Configuration -------------------------------- Auto start : Enabled Max Traffic MODCOD : 23 16APSK-9/10 RX watchdog : 15 minute Idx Pri SymbRate[Msps] Freq[GHz] Mode PopId SatId Pos SatName Name Enable * 0 0 25.000000 11.628079 DVB-S2 105 0 0.0 E Yes Satellite (DVB) Receiver Status -------------------------------- Rx State : On DVB State : Forward link up Network : 1326, ATI-PI Frequency : 11.628109 GHz Symbol Rate : 25.000115 Msps S2 ModCod - receiving : 1 QPSK-1/4 - current max : 14 8PSK-3/4 Pilot : On Frame length : Short DVB S2 Mode : ACM DVB S2 Stream type : MPEG-TS Roll off : 0.25 SNR : 9.9 dB Input Power : -40 dBm # arp show NetAddr State MacAddr If 10.225.1.98 Dynamic 00:90:27:f0:30:da 1 # ip show Interfaces If IPAddress SubnetMask BroadCastAddr MTU Alias AdminStatus 1 10.225.1.97 255.255.255.252 10.225.1.99 1500 eth0 1 3 172.18.193.221 255.255.255.255 N/A 1500 dvb0 1 4 N/A N/A N/A 1500 air0 1 DNS Client Configuration Primary DNS Server: 200.215.179.99, Secondary DNS Server: 200.215.179.97 Interface Statistics ------------- Input ----------------- ------------- Output ---------------- If UCast NUCast Disc Octets UCast NUCast Disc Octets 1 157079 0 59 45536073 241607 40 0 268167757 3 240541 0 0 269777112 94240 0 704 38966934 4 0 0 0 0 0 0 0 0 IP Receive Deliver Errors Discards Forward Request NoRoute Discards 397420 129 0 0 336293 196 0 97 Interface Directed Routing Table In If Out If ---------------IP LAC Statistics---------------- RPF Src eq dest RPF HitCount SrceqdestHitCount MfcHitCount (-1/drop classified) Disabled Enabled 0 0 0 Routing Table DestMask RouteMask NextHop If RtType 0.0.0.0 0.0.0.0 0.0.0.0 3 STATIC 10.225.1.96 255.255.255.252 0.0.0.0 1 LOCAL 10.225.98.0 255.255.255.0 10.225.1.98 1 STATIC 172.18.193.221 255.255.255.255 0.0.0.0 3 LOCAL # |
| 3 – Configurações dos Equipamentos 4 – Teste de Velocidade Download |  |  |  |  |  |