|  |  |  | TERMO DE ACEITE TÉCNICO |  |  |
| --- | --- | --- | --- | --- | --- |
| Solicitante: ATI - Agência de Tecnologia da Informação do Estado do Piauí |  |  |  |  |  |
| NOC/Operadora: Leonardo Martins SIlva |  |  |  | Técnico Instalador: Claudinei Oliveira Contato: 11 97701-2709 Email: claudnei@telecomtorre.com.br |  |
|  |  |  |  | Responsável/cliente: |  |
| Orgão: SEDUC | Unidade:Produtiva Familiar Associação Roça de Milho |  |  | Data Instalação 23/09/2018 | Data Ativação 23/09/2018 |
| Operadora Global Eagle |  | VSAT-ID ATI-OU-0034-IND |  | Porta kbps 1124/256 | Tipo Indoor |
| Piauí, 23 de Setembro de 2018 1 – Local da Instalação. 2 – Configurações do Modem # device show Configuration Saved System Information: Name : ATI-OU-0034-IND Location : Aroazes - PI Contact : System Up time : 0 days, 00:59:23 CPU Load : 99% System time(UTC) : 23 September 2018 12:51:25 Broadcast Message : not set HW: Model : SatLink 2000 HW ID : 120033 Main board ID : 120026 R6.2 MAC addresses: LAN : 00:20:0e:10:8c:00 Satellite (DVB) : 00:20:0e:10:8c:00 # dvb tx show Satellite (DVB) TX Configuration -------------------------------- Auto start : Enabled BUPC status : Disabled IDU Output Power : -15 dBm IDU Max Output Power: 0.0 dBm Default CW Frequency: 0.000000 GHz ATM mode : VC-Mux Header Compression : None Satellite (DVB) Transmitter Status -------------------------------- State : On (DVB-RCS2) IDU Output Power : -15.2 dBm Es/No : 14.0 dB Header Compression : Disabled Timing correction : -272 us (249802 us) Frequency correction: 180 Hz # dvb rx show Satellite (DVB) RX Configuration -------------------------------- Auto start : Enabled Max Signalling MODCOD : 23 16APSK-9/10 Max Traffic MODCOD : 28 32APSK-9/10 RX watchdog : 15 minute Idx Pri SymbRate[Msps] Freq[GHz] Mode PopId SatId Pos SatName Name Enable * 0 0 25.000000 11.628079 DVB-S2 105 0 0.0 E Yes Satellite (DVB) Receiver Status -------------------------------- Rx State : On DVB State : Forward link up Network : 1326, ATI-PI Frequency : 11.628094 GHz Symbol Rate : 24.999818 Msps S2 ModCod - receiving : 1 QPSK-1/4 - current max : 15 8PSK-5/6 Pilot : On Frame length : Short DVB S2 Mode : ACM DVB S2 Stream type : MPEG-TS Roll off : 0.25 SNR : 13.3 dB Input Power : -35 dBm # odu show Antenna --------------------- Type ASC/Andrew/Channel Master Type 123 - 1.2m Antenna controller None Tx Gain 43.3dB Transmitter (BUC) ------------------------------- Type JRC NJT5017 (14.0-14.5 GHz) Local oscillator 13.050000 GHz DC supply 24V On Reference 10Mhz On Receiver (LNB) ---------------------------- Type Zinwell ZK-VJ1 (10.70-12.75 GHz) Local oscillator - LO1 9.750000 GHz Local oscillator - LO2 10.600000 GHz Oscillator switching frequency 1-2 11.700000 GHz LO Switching mode 22KHz 13/18V DC supply 13V # ip show Interfaces If IPAddress SubnetMask BroadCastAddr MTU Alias AdminStatus 1 10.225.0.177 255.255.255.252 10.225.0.179 1500 eth0 1 3 172.18.192.59 255.255.255.255 N/A 1540 dvb0 1 4 N/A N/A N/A 1500 air0 1 DNS Client Configuration Primary DNS Server: 209.244.0.3, Secondary DNS Server: 209.244.0.4 Interface Statistics ------------- Input ----------------- ------------- Output ---------------- If UCast NUCast Disc Octets UCast NUCast Disc Octets 1 257 14 20 30186 239 33 0 58026 3 343 0 0 64445 400 0 0 36896 4 0 0 0 0 0 0 0 0 IP Receive Deliver Errors InDiscards Forward Request NoRoute OutDiscards 580 159 0 0 678 245 0 36 Interface Directed Routing Table In If Out If ---------------IP LAC Statistics---------------- RPF Src eq dest RPF HitCount SrceqdestHitCount MfcHitCount (-1/drop classified) Disabled Enabled 0 0 0 Routing Table DestMask RouteMask NextHop If RtType Row-Status 0.0.0.0 0.0.0.0 0.0.0.0 3 STATIC ACTIVE 10.225.0.176 255.255.255.252 0.0.0.0 1 LOCAL ACTIVE 10.225.54.0 255.255.255.0 10.225.0.177 1 STATIC ACTIVE 172.18.192.59 255.255.255.255 0.0.0.0 3 LOCAL ACTIVE # 3 – Configurações dos Equipamentos |
| 4 – Teste de Velocidade Download/Uploand |  |  |  |  |  |